EE 3755    Homework 3      Due: March 28,2005

Estimated time to solve:

Prob.1        5 mins.

Prob.2         2 mins.

Prob.3         4hours.

Prob.4         2 hours.(check one of the previous homework then it will be 30mins)

Prob.5         3 mins..

Total:          From 4hour 40mins to 6hour 10mins.

How to submit? // Hard copy at the class room.

For Problem 2,3,and 4 ,you also have to leave your program on your account.

Please write down actual time you spend to do the each problem.

Use “script “ command to take a snap shot of your program.

After run “script “

Use “cat” command to display  contents of  your program.

Use “ncverilog” to run your program.

then stop the script.

For Prob.2 to Prob. 4,  take a snapshot and submit the snapshot.

Problem 1: Convert the following numbers. (5mins)

Hint (7/8) = 0.875

a) Decimal  12  to 8-bit Binary:

b) Decimal  -12     to 8-bit Binary:

c) Decimal 12.875  to Binary (as many bits as needed):

d) Decimal    12.875  to IEEE 754 Single Precision (Show in hexadecimal):

Problem 2 : Show the output of the module.

module behavioral_1(x);

output x;

reg [7:0]  x;

initial

// Procedural Code Starts Here

begin

x = 1;

$display("Hello, x=%d, t=%t",x,$time);

x = 2;

$display("Hello, x=%d, t=%t",x,$time);

x = 3;

$display("Hello, x=%d, t=%t",x,$time);

end

endmodule

//Ans.

// Simulator Output

//

module behavioral_2(x);

output x;

reg [7:0]  x;

initial

begin

x = 1;

$display("Hello, x=%d, t=%t",x,$time);

#1;

x = 2;

$display("Hello, x=%d, t=%t",x,$time);

#1;

x = 3;

$display("Hello, x=%d, t=%t",x,$time);

#1;

end

endmodule

// Ans.

// Simulator Output

//

Problem 3 : Write a simple Verilog program which will convert

a decimal number to IEEE 754 single format number

and  test the solution of problem 1.d.(12.875)//

your program should print 32bit binary number.

Output format should be this:

## start of output

Decimal -10.625  convert to IEEE 754 Single Precision

1100 0001 0010 1010 0000 0000 0000

##end of output

hint:

module test;

real float,float2; // Like a float in C

//1)you also have to declare a data type for the output //here

initial

begin

float = 12.875;

float2 = 12;

$display ("%f\n",float);

$display ("%f\n",float-float2);

i = float;

$display ("%f\n",i);

//

//2)you have to write conversion code here and print

//the result,  run the program and get  a  snapshot of it.

Problem 4 : Write a population count program and a test bench which will generate this following output.

//you have to write a program and take snapshot of it.

//out put solution is following

time  input                         output

100 input a = 00000000000000000000000000000000 output p =  0

200 input a = 00000000000000000000000000000001 output p =  1

300 input a = 00000000000000000000000000000010 output p =  1

400 input a = 00000000000000000000000000000011 output p =  2

500 input a = 00000000000000000000000000000100 output p =  1

600 input a = 00000000000000000000000000000101 output p =  2

700 input a = 00000000000000000000000000000110 output p =  2

800 input a = 00000000000000000000000000000111 output p =  3

900 input a = 00000000000000000000000000001000 output p =  1

1000 input a = 00000000000000000000000000001001 output p =  2

1100 input a = 00000000000000000000000000001010 output p =  2

1200 input a = 00000000000000000000000000001011 output p =  3

1300 input a = 00000000000000000000000000001100 output p =  2

1400 input a = 00000000000000000000000000001101 output p =  3

1500 input a = 00000000000000000000000000001110 output p =  3

1600 input a = 00000000000000000000000000001111 output p =  4

Problem 5 : Answer the sub problems.

5-1)Which code will be longer ? Structural or Implicit ?(Hint : Gates)

(Estimated Time to Finish:  30s.)  .

Answer:

5-2)Fill  the blanks.( Estimated Time to Finish:  30s.)

| a, b | a & b | a && b |
| --- | --- | --- |
| 000 000 |  |  |
| 010 111 |  |  |

(Hint:: Arithmetic, logical)

5-3)Fill the blanks; x1 = a == b;

( Estimated Time to Finish:  30s.)

| a, b | x1 = a== b |
| --- | --- |
| 000 000 |  |
| x01 001 |  |

5-4)Fill the blanks x = {a, b};

( Estimated Time to Finish:  30s.)

| A, b | x = {a,b}; |
| --- | --- |
| 000 000 |  |
| 001 111 |  |

5-5)Fill the blanks x = a >> 2;

( Estimated Time to Finish:  30s.)

| A | x = a >> 2 |
| --- | --- |
| 1100 |  |
| 0100 |  |

5-6) Write down the value.

//a = 10;

x =   a < 12 ?  10 :  20;

What is the x value?(10 or 20).