EE3755  Homework 3  Due :TBA

Estimated time to finish:

Prob.1:  30Mins.

Total :   30Mins.

Use bgx_shell –gui

How to submit: Hard copy during the class.

Problem 1. Use the Synthesis program.

1.1) Print out or Draw the figure for the module(90pts)

1.2) Look at the figure and briefly explain the functionality of the circuit.(10pts)

module neq_slice_es(eo,a,b,ei);

input a, b, ei;

output eo;

wire   aneb, aeqb;

xor x1(aneb,a,b);

and a1(eo,aneb,ei);

endmodule

module nequal(eq,a,b);

input [3:0] a, b;

output      eq;

wire        e1, e2, e3;

neq_slice_es es3(e3,a[3],b[3],1'b1);

neq_slice_es es2(e2,a[2],b[2],e3);

neq_slice_es es1(e1,a[1],b[1],e2);

neq_slice_es es0(eq,a[0],b[0],e1);

endmodule