EE 3755    Homework 1      Due: October 14, 2009

Estimated time to solve:

Prob.1         30 minutes.

Prob.2         30 minutes.

Prob.3         30 minutes.

Prob.4         30 minutes.

Preparing text files 1 hour.

Total           3 hours.

How to submit?

//  No hard copy.

//  Name your code file as hw1.v

//  Name your script file   as hw1.txt

//  Leave your code file(hw1.v) and

//  your script  file(hw1.txt) on the class account.

Finish your code file(hw1,v) first, then do the followings:

Use “script  hw1.txt “ command // To take a snap shot of your code. If you don’t know

// script command, look at the handout.

Use “cat  hw1.v” command      // To display  contents of  your code.

Use “ncverilog hw1.v”           // To run your code.

//  stop the script.

Example)

SUN>script  hw1.txt

SUN>cat hw1.v

SUN>ncverilog hw1.v

SUN> ( stop  the script  by pressing CTRL-D)

Fig.1 One_Bit _Not_Equal_Check_with_Enable_Low Slice. (one AND gate, one OR gate, one XOR gate, one NOT gate ; four Inputs and one Output)      //Circuit for the Problem 1.

Fig.2  Two One_Bit_Not_Equal_Check_with_Enable_Low Slices( cascaded for the problem 3.)

Problem 1:   Complete module  one_bit_not_equal_check_with_enable_low_slice_ex (in the homework template) so that it is an explicit structural description of the circuit  illustrated at Fig.1.  Remember that this module only handles one bit of  A and B.

Problem 2:   Complete module one_bit_not_equal_check_with_enable_low_slice_im so that it is an implicit structural description of the circuit  illustrated at Fig.1.  Remember that this module only handles one bit of A and B.

Problem 3:   Complete module two_one_bit_not_equal_check_with_enable_low_slice_ex illustrated at Fig.2  by using  two one_bit_not_equal_check_with_enable_low_slice_ex modules and explicit structural descriptions.

Problem 4:: Module three_bit_equal_position_detector  is used to detect bit position where the two input bits  are equal  for the first time.

It has two three-bit inputs( a and b),  and  three one-bit outputs ( Position_1,Position_2,Position_3 ). The code for the one_ bit _ equal _position_detector  is given.

Complete the module three_bit_equal_position_detector  by using three one-bit-equal _positon_detector modules.

For example)  if three-bit input  a=111, b=111 then  Position = 100(position_1, position_2, position_3)

if three-bit input  a=011, b=111 then  Priority = 010(position_1, position_2, position_3)

if three-bit input  a=010, b=101 then  Priority = 000(position_1, position_2, position_3)

///#### LSU EE 3755  Fall 2009 Verilog  Homework 1- Code  template.#############

///#### Instructions:

///####

///#### Copy this to a file( name it  hw1.v) and save on  your class account.

///#### Use this file for your solution.

///#### Your entire solution should be on this file.

///#### Do not rename the modules in this file and be sure to use the file name given above.

///

////////////////////////////////////////////////////////////////////////////////

/// Problem 0

module hello();

initial begin

//#### Display your name and class account.

//#### Write your code here.

//#### For example

//#### If your name is "Clark Kent" and class account is ee375501

//#### The exact answer will be:

$display("ee375501 Clark Kent\n");//simply change the account number and  name.

end

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 1

module one_bit_not_equal_check_with_enable_low_slice_ex(ab_next,ab_prev,enable_low,a,b);

input   ab_prev,enable_low,a,b;

output ab_next;

wire   xor_ab, not_enable_low, or_abprev_abxor;

//##### your solution goes here

//

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 2

module one_bit_not_equal_check_with_enable_low_slice_im(ab_next,ab_prev,enable_low,a,b);

input   ab_prev,enable_low,a,b;

output ab_next;

wire   xor_ab, not_enable_low, or_abprev_abxor;

//##### your solution goes here

//

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 3

module two_one_bit_not_equal_check_with_enable_low_slice_ex (ab_next_1,ab_prev_0,enable_low,a,b);

input [1:0]  a, b;

input ab_prev_0,enable_low;

output ab_next_1;

//#### your solution goes here

//#### 2 lines.

endmodule

/// Problem 4

module one_bit_equal_position_detector(P_set_i_1, Position,a,b,P_set_i);

input  a,b,P_set_i;

output P_set_i_1,Position;

reg    P_set_i_1,Position;

always  begin

P_set_i_1  = 0;

Position   = 0;

if((a == b) & (P_set_i == 0)) begin

P_set_i_1  = 1;

Position = 1;

end

#5;

if(P_set_i ==1) begin

P_set_i_1 = 1;

//Position =0;

end

#5;

end

endmodule

//complete this module

module three_bit_equal_position_detector(Position,  a, b);

input [2:0]  a, b;

output[2:0] Position;

wire [2:0]  P_set;

//#### Your solution goes here

//####  3 lines

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Test Bench

/// Do not modify test bench

module test();

wire[2:0]  position;

reg [2:0]  a, b;

integer    i;

three_bit_equal_position_detector  com1(position ,a,b);

initial begin

for(i=0; i<=63; i=i+1) begin

a = i[2:0];

b = i[5:3];

#100;

$display("a = %b,b=%b,position=%b\n",a,b,position);

end

$display("Tests completed.");

$finish;

end

endmodule

//########## Hint for problem 4############################################

//## When you run your code, you will see these from the output.

//##////  a = 000,....

//##////       ......

//##////  a = 011,b=111,position =  010

//##////  a = 100,b=111,position =  100

//##////  a = .....

//#########################################################################