EE 3755    MIPS 1      Due: TBA

.

Problem 1: Write a MIPS program to multiply two 8byte numbers.

// a = {$t1,$t2}

//  b = {$t3,$t4}

//  c= {$s1, $s2,$s3,$s4} = a*b;

//  For testing purpose give these values to registers.

// a = 0x1111123411111234.

// b = 0x1111123411111234.

8byte long  number a is stored at $t1,$t2.

8byte long  number b is stored at $t3,$t4.

Store the result at $s1,$s2,$s3,$s4.

  - Save your program on your account and name it mips1.asm.

  - Also submit a hard copy.

Problem 2: Answer the questions without looking anything.(this is practice, so do not use the simulator program although you may check the correctness of your answer)

LOOP:

0x400000        add r1, r2, r4

addi r1, r2,0x20

nop

beq r0,  r0, LOOP

(For the problem 2.1)

2.1)Write Machine code for the above 4 instructions.

(Hexadecimal format)

Hint :

opcode for add  is 0 and func field is 0x20.

opcode for addi is 8 and think about rt and rs fields.

opcode for beq  is 4

What will be the offset value for LOOP.

##############################################

START:

0x400000    J LINE1

Nop

Nop

LINE1       beq r0,r0,LINE2

add r1,r0,r0

LINE2       nop

(For the problem 2.2)

2.2)Write Machine code for the above 5 instructions.

(Hexadecimal format)

Hint :

opcode for j is 2 .

What will be the offset value for LINE1 and LINE2.

2.3)Compute this branch target address:

0x400000    bne r1, r2, 3

2.4)compute this branch target address

0x40000c    bne r1,r2,-3

Problem 3: Briefly explain about three instruction formats for MIPS.

(Less than 20lines.

Your answer should contain at least these following information.

- a)      How big is each field at each format?

- b)      Field position.

Problem 4. write the  register values after executing following instructions.

Loop:

| 0x1000 | Addi r1, r0, 3; | R1= |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Addi r2, r0, 5; | R1= | R2= |  |  |  |
|  | Sub r3, r1, r2; | R1= | R2= | R3= |  |  |
|  | Sub r4, r1, r2; | R1= | R2= | R3= | R4= | R31= |
|  | Beq r3, r0, loop; |  |  |  |  | R31= |
|  | ori r1, r2, 20; | R1= | R2= | R3= |  | R31= |
|  | Addi r4, r0, 0x2000; | R1= | R2= | R3= | R4= | R31= |
|  | jalr r4; | R1= | R2= | R3= | R4= | R31= |

Problem 5: True(right) or False(wrong) instruction .

T      F

5-1)  addi $t0,0x1234,$t0

5-2)  subi $t0,$t1,0x1234

5-3)  jr    $0x123456

5-4)  j     0x123456789

Problem 6:  Briefly explain about the register usage for procedure call.

(Less than 20 lines, please. your answer at least should contain explanation about v0, a0,t0,s0 registers)