EE 3755    Homework 4      Due: Nov.3

Estimated time to solve:

Prob.1            30  mins.

Prob.2          120 mins.

Prob.3          240 mins

Prob.4          120 mins

Total:         .

Please write down actual time you spend to do the each problem.

Problem 1: Convert the following numbers.

1.1) Decimal    11      to 8-bit Binary:

1.2) Decimal   -11      to 8-bit Binary:

1.3) Decimal    11.875  to Binary (as many bits as needed):

1.4) Decimal   -11.875  to IEEE 754 Single Precision

(Show in hexadecimal):

Problem 2:   Add these two numbers:

A = 1.5

B = 1.5

2.1)  Convert A to IEEE 754 Single precision //

2.2)  Use module fp_add(sum, exp_overflow, a_original,b_original)

from the Notes 10  to add A and B;

Show the output by using these lines:

$display(" sum = %h\n",sum);

$display(" sum = %d\n",sum);

Submit the output and the testbench program(around 10 lines).

2.3)  Decimal output for the second line is

not 3,although 1.5 + 1.5 = 3.0.

Why?

Problem 3: To fix the 2.3 problem, Write a conversion module which will take

input in  IEEE 754 format  and produce output in decimal format.

dec_conversion(dec_sum,sum);

3.1) Show the output by using these lines:

$display(" sum = %h\n",sum);

$display("decimal sum = %d\n",dec_sum);

3.2) Submit the output and the code for the conversion module.

3.3) Name your program hw33.v and leave it on your account.

Problem 4: Add these two numbers:

A = 1.5;

B = 1.5;

Use module

fp_add_seq(sum, exp_overflow,ready,a_original,b_original,start,clk)

from Notes 10 to add A and B;

Use clk which has #1 delay.

// #1;

// clk =~clk;

4.1)  Show the output by using this line:

$display(" sum = %h\n",sum);

4.2)  Submit the testbench program( around 25 lines).

4.3)  True or False.

since A and B are 32 bits long, the sequential floating point adder

,fp_add_seq(sum,exp_overflow,ready,a_original,b_original,start,clk),

will take at least  32 clks to produces right output.

4.4)  Bonus Points Problem( 7 pts):

the sequential floating point adder,

fp_add_seq(sum,exp_overflow,ready,a_original,b_original,start,clk),

will produce right output after (  ) cycles.