EE 3755    Homework 3      Due: TBA

Estimated time to solve:

Prob.0         60mins.

Prob.1         60 mins.// fill out the program file, leave

Prob.2         60 mins.// on your directory and submit

Prob.3         60 mins.// a hard copy.

Prob.4         60 mins

Prob.5         60 mins

Total:         .

How to submit? // Hard copy at the class room. It should contain the output

Of problem 5.

#########################################################

###Use “script “ command to take a snapshot of your program.

###After run “script “

###Use “cat” command to display  contents of  your program.

###Use “ncverilog” to run your program.

###then stop the script.

### if you want to know more about “script” then type “man script” at the shell

#Name your program hw3.v

Problem 0:  run the code below ( "memory" and "test")

and look at the output.

module memory();

reg [3:0] mem [0:15];

integer    i;

initial

begin

for(i=0;i<16;i = i+1) mem[i] = i;

end

endmodule

module test();

memory m();

initial begin

for(i=0;i<16;i = i+1) $display(" mem[%d] = %b\n",i,m.mem[i]);

end

endmodule

///////////////////////////////////////////////////////////////

Problem 1:

Write a program which will read 4bits from the memory at every clock cycle

starting at address 0 and go up to address 15 and go back address 0 again.

///////////////////////////////

module memory_read_c(v,ck);

input ck;

output v;

//your code comes here (about 10 lines).

endmodule

///////////////////////////////

Problem 2:

Write a program which will read 1bit from the memory at every clock cycle

starting at address 0's LSB, next bit, next bit, MSB  and go up to address 15

and go back address 0 again (reading one bit at a time).

////////////////////////////////

module memory_read_s(v,ck);

input ck;

output v;

////////////////////////

//your solution comes here (about 12 lines)

endmodule

//////////////////////////////////

Problem 3:

Write a program which will read 4bits from the memory at every clock cycle

and test if the number of 1s and 0s are equal or not.

starting at address 0 and go up to address 15 and go back address 0 again.

//////////////////////////////////

//  memory_contents  same

//       0000        0

//       0001        0

//       0010        0

//       0011        1

//       xxxx        x

//////////////

module test_memory_c(same,ck);

input ck;

output same;

reg same;

////////

your solution comes here

endmodule

////////////////

Problem 4:

Write a program which will read 1bits from the memory at every clock cycle

and test if the number of 1s and 0s are equal or not at every     4 cycles.

starting at address 0 and go up to address 15 and go back address 0 again.

////////////////////////////////////

module test_memory_s(same,ck);

input ck;

output same;

reg same;

///////////

your solution comes here

endmodule

////////////////

Problem 5.

After you finish the problem 1 to the problem 4,

Run  below test_all module for each of your solution for testing.

module test_all();

integer i,j;

reg ck;

wire [3:0] v1;

wire vs;

wire same;

// memory m();                  for the problem 0.

// memory_read_c m_r(v1,ck);    for the problem 1.

// memory_read_s m_r_s(vs,ck);  for the problem 2.

// test_memory_c t_c(same,ck);   for the problem 3.

// test_memory_s t_s(same,ck);   for the problem 4.

initial  ck = 0;

initial begin

for(i = 1; i<= 20; i = i+ 1)

begin

#1;

ck = ~ck;

end

end

always @(ck)

begin

//       $display("problem 1 time = %t , v1 = %b\n",$time,v1);

// for the problem 1.

//       $display("problem 2 time = %t , v1 = %b\n",$time,vs);

// for the problem 2.

//       $display("problem 3 time = %t , same = %b\n", $time,same);

//for the problem 3.

//       $display("problem 3 time = %t , same = %b\n", $time,same);

//for the problem 4.

end

endmodule