EE 3755    Verilog Homework 1      Due: Oct 21, 2011

Estimated time to solve:

Prob.1         30 minutes.

Prob.2         30 minutes.

Prob.3         30 minutes.

Prob.4         30 minutes.

Preparing text files 1 hour.

Total           3 hours.

How to submit?

//  No hard copy.

//  Name your code file as hw1.v

//  Name your script file   as hw1.txt

//  Leave your code file(hw1.v) and

//  your script  file(hw1.txt) on the class account.

Finish your code file(hw1,v) first, then do the followings:

Use “script  hw1.txt “ command // To take a snap shot of your code. If you don’t know

// script command, look at the handout.

Use “cat  hw1.v” command     // To display  contents of  your code.

Use “ncverilog hw1.v”          // To run your code.

then stop the script.

Example)  SUN>script  hw1.txt

SUN>cat hw1.v

SUN>ncverilog hw1.v

SUN>  ( stop script  by pressing CTRL-D)

Fig.1 One_Bit_OR  Slice.

(one_AND gate,one_OR gate,one NOT gate; four inputs and two outputs)

//Circuit for the Problem 1.

Fig.2  Two One_Bit_OR Slices( cascaded for the problem 3.)

Fig.3  One_Bit_Value_Checker Slice.( Cascadable for the problem 4)

This one has 3 one-bit- inputs(Ai, Bi,and Flag_in)

and 2 one-bit-outputs(P and Flag_out).

P = 1 when (Ai=1 and Bi =0 and Flag_in =1) otherwise  0.

Flag_out = 0 when (Ai=1 and Bi =0 and Flag_in =1) otherwise 0.

Problem 1:   Complete module  one_bit_OR_slice_ex (in the homework template) so that it is an explicit structural description of the circuit  illustrated at Fig.1.

Remember that this module only handles one bit of  A and B.

Problem 2:   Complete module one_bit_OR_slice_im so that it is an implicit structural description of the circuit  illustrated at Fig.1.

Remember that this module only handles one bit of A and B.

Problem 3:  Complete module two_one_bit_OR_slices  illustrated at Fig.2  by using  two one_bit_OR_slice_im modules and explicit structural descriptions.

Problem 4: Module three_bit_value_checker  is used to look two three-bit numbers.

It has two three-bit inputs( a and b),  and  three one-bit outputs(p0,p1,p2).

The code for the one- bit-value_checker is given.

It will detect the bit position where the input value for a is '1' and b is '0' for the first time.

Complete the module three_bit_value_checker  by using three one_bit_value_checkers.

For example)   if three-bit inputs  a=111(a0,a1,a2), b=111(b0,b1,b2)  then p0,p1,and p2 will be 0,0,0.

if three-bit inputs  a=111(a0,a1,a2), b=110(b0,b1,b2)  then p0,p1,and p2 will be 0,0,1

if three-bit inputs  a=111(a0,a1,a2), b=100(b0,b1,b2)  then p0,p1,and p2 will be 0,1,0

if three-bit inputs  a=111(a0,a1,a2), b=000(b0,b1,b2)  then p0,p1,and p2 will be 1,0,0

if three-bit inputs  a=000(a0,a1,a2), b=100(b0,b1,b2)  then p0,p1,and p2 will be 0,0,0

///#### LSU EE 3755  Fall  2011 Verilog  Homework 1- Code  template.#############

///#### Instructions:

///####

///#### Copy this to a file( name it  hw1.v) and save on  your class account.

///#### Use this file for your solution.

///#### Your entire solution should be on this file.

///#### Do not rename the modules in this file and be sure to use the file name given above.

///

////////////////////////////////////////////////////////////////////////////////

/// Problem 0

module hello();

initial begin

//#### Display your name and class account.

//#### Write your code here.

//#### For example

//#### If your name is "Clark Kent" and class account is ee375501

//#### The exact answer will be:

$display("ee375501 Clark Kent\n");//simply change the account number and  name.

end

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 1

module one_bit_OR_slice_ex(p_out,flag_out,a,b,c,flag_in);

input a, b,c,flag_in;

output p_out,flag_out;

wire and_abc,or_out;

// your solution goes here

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 2

module one_bit_OR_slice_im(p_out,flag_out,a,b,c,flag_in);

input a, b,c,flag_in;

output p_out,flag_out;

wire and_abc,or_out;

// your solution goes here

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 3

module two_one_bit_OR_slices (p_out,flag_out_1,a,b,c,flag_in_0);

input [1:0]  a, b,c;

input flag_in_0;

output flag_out_1;

output [1:0] p_out;

wire   flag_out_0;

// your solution goes here

// less than 3 lines.

endmodule

////////////////////////////////////////////////////////////////////////////////

////////////////////////////////////////////////////////////////////////////////

//// Problem 4

module one_bit_value_checker(P,Flag_out, a, b, Flag_in);

input a, b,Flag_in;

output Flag_out,P;

reg Flag_out,P;

always  begin

Flag_out =0;

P = 0;

if((a == 1)& (b==0) & (Flag_in == 0))  begin

Flag_out = 1;

P = 1;

end

if(Flag_in ==1) begin

Flag_out = 1;

P=0;

end

#1;

end

endmodule

//complete this module

module three_bit_value_checker(P, a, b);

input [2:0]  a, b;

output[2:0] P;

wire  Flag_1,Flag_2,Flag_3;

//Your solution goes here

// 3 lines

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Test Bench

/// Do not modify test bench

module test();

wire[2:0]  position;

reg [2:0]  a, b;

integer    i;

three_bit_value_checker  com1(position ,a,b);

initial begin

for(i=0; i<=63; i=i+1) begin

a = i[2:0];

b = i[5:3];

#10;

$display("a = %b,b=%b,position=%b\n",a,b,position);

end

$display("Tests completed.");

end

endmodule