EE 3755    Homework 1      Due: March 8

Spring 2010

Estimated time to solve:

Prob.1         30 minutes.

Prob.2         30 minutes.

Prob.3         30 minutes.

Prob.4         30 minutes.

Preparing text files  1 hour.

Total           3 hours.

How to submit?

// No hard copy.

// Name your homework file as hw1.v

// Name your script file  as hw1.txt

// Leave your homework file(hw1.v) and

// your script file(hw1.txt) on the class account.

Finish your homework  file(hw1,v) first, then do the followings:

Use “script  hw1.txt “ command  // To take a snap shot of your code. If you don’t know

// script command, look at the handout.

Use “cat  hw1.v” command       // To display  contents of  your code.

Use “ncverilog hw1.v”             //To run your code.

then stop the script.

Example)  titan>script  hw1.txt

titan>cat hw1.v

titan>ncverilog hw1.v

titan> ( stop script  by pressing CTRL-D)

Fig.1 One_Bit_Slice(one AND gate, one XOR gate, one NOT gate; four inputs and one output).

//Circuit for the Problem 1.

Fig.2  Two One_Bit_Slices( cascaded for the problem 3.)

Fig.3  One Bit Detect  Slice.( Cascadable for the problem 4)

This one has 3 one-bit- inputs(Ai, Bi,and P_set_i)

and 2 one-bit-outputs(Position_i and P_set_i+1).

Position_i = 1  when (Ai = 1, Bi=1   and P_set_i =0), otherwise 0.

P_set_i+1  = 1  when Position_i =1, otherwise 0.

Problem 1:   Complete module  one_bit_slice_ex (on the homework template) so that it is an explicit structural description of the circuit  illustrated at Fig.1.  Remember that this module

only handles one bit of  A , B and C.

Problem 2:   Complete module one_bit_slice_im so that it is an implicit structural description of the circuit  illustrated at Fig.1.  Remember that this module only handles one bit of A , B and C.

Problem 3: Complete module two_one_bit_slices  illustrated at Fig.2  by using  two one_bit_slice_ex modules and explicit structural descriptions.

Problem 4: Module three_bit_detect_unit  is used to detect bit position where the two input bits are 1 for the first time.

It has two three-bit inputs( a and b),  and  three one-bit outputs ( Position_1,Position_2,Position_3 ). The code for the one_bit_detect_unit  is given.

Complete the module three_bit_detect_unit  by using three one_bit_detect_unit  modules.

For example)  if three-bit input  a=111, b=111 then  Position = 100(position_1, position_2, position_3)

if three-bit input  a=011, b=111 then  Position = 010(position_1, position_2, position_3)

if three-bit input  a=010, b=101 then  Position = 000(position_1, position_2, position_3)

///#### LSU EE 3755  Spring  2010 Verilog  Homework 1- Code  template.#############

///#### Instructions:

///####

///#### Copy this to a file( name it  hw1.v) and save on  your class account.

///#### Use this file for your solution.

///#### Your entire solution should be on this file.

///#### Do not rename the modules in this file and be sure to use the file name given above.

///

////////////////////////////////////////////////////////////////////////////////

/// Problem 0

module hello();

initial begin

//#### Display your name and class account.

//#### Write your code here.

//#### For example

//#### If your name is "Clark Kent" and class account is ee375501

//#### The exact answer will be:

$display("ee375501 Clark Kent\n");//simply change the account number and  name.

end

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 1

module one_bit_slice_ex(abc_next,abc_pre,a,b,c);

input  a, b, c, abc_pre;

output abc_next;

wire   abc_xor, abc_pre_not;

//##### your solution goes here

//#####   3 lines

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 2

module one_bit_slice_im(abc_next,abc_pre,a,b,c);

input  a, b, c ,abc_pre;

output  abc_next;

wire   abc_xor, abc_pre_not;

//#### your solution goes here

//#### could be only 1 line

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 3

module two_one_bit_slices (abc_next,a,b,c,abc_pre);

input [1:0]  a, b,c;

input abc_pre;

output abc_next;

wire   abc_1;

//#### your solution goes here

//#### 2 lines.

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Problem 4

module one_bit_detect_unit(P_set_i_1, Position,a,b,P_set_i);

input  a,b,P_set_i;

output P_set_i_1,Position;

reg    P_set_i_1,Position;

always  begin

P_set_i_1  = 0;

Position   = 0;

if( (a == 1) & (b == 1) & (P_set_i == 0)) begin

P_set_i_1  = 1;

Position = 1;

end

#5;

if(P_set_i ==1) begin

P_set_i_1 = 1;

end

#5;

end

endmodule

//complete this module

module three_bit_detect_unit(Position,  a, b);

input [2:0]  a, b;

output[2:0] Position;

wire [2:0]  P_set;

//#### Your solution goes here

//####  3 lines

endmodule

////////////////////////////////////////////////////////////////////////////////

/// Test Bench

/// Do not modify test bench

module test();

wire[2:0]  position;

reg [2:0]  a, b;

integer    i;

three_bit_detect_unit  com1(position ,a,b);

initial begin

for(i=0; i<=63; i=i+1) begin

a = i[2:0];

b = i[5:3];

#100;

$display("a = %b,b=%b,position=%b\n",a,b,position);

end

$display("Tests completed.");

$finish;

end

endmodule

//########## Hint for problem 4############################################

//## When you run your program, you will see these from the output.

//##////  a = 000,....

//##////       ......

//##////  a = 011,b=111,position =  010

//##////  a = 100,b=111,position =  100

//##////  a = .....

//#########################################################################