EE 3755    Homework 2      Due: March. 25.

Estimated time to solve:

| Prob.1 |  | 5 | minutes | (15pts) |
| --- | --- | --- | --- | --- |
| Prob.2 |  | 10 | minutes | (10pts) |
| Prob.3 |  | 7 | hours | (65pts) |
| Having Script File |  | 5 | minutes | (10pts) |
| Total |  | 7hours and 10minutes |  |  |

How to submit?

//  No hard copy.

//  Name your solution file as hw2.v

//  Name your script file   as hw2.txt

//  Leave your solution file(hw2.v) and

//  your script  file(hw2.txt) on the class account.

Finish your solution file(hw2.v) first, then do the followings:

Use “script  hw2.txt “ command // To take a snap shot of your code. If you don’t know

// script command, look at the handout.

Use “cat  hw2.v” command     // To display  contents of  your code file.

Use “ncverilog hw2.v”           //To run your code.

then stop the script.

Example) SUN>script  hw2.txt

SUN>cat hw2.v

SUN>ncverilog hw2.v

SUN>  ( stop script  by pressing CTRL-D)

#Do all your work on hw2.v file

After carefully read the problem description, write code on the template file(hw2.v).

Fig.1.  Mini_serial_processing_unit.

One bit serial input and 6 bit output.

## Problem description##

You are required to build a mini serial processing unit.  module mini_serial _processing_unit(vout,v,ck);

The unit will receive  bit-stream serially and each bit is arrived  at every clock cycle.

After receiving certain bit pattern(operator), you have to process the incoming bit- stream accordingly.

To do this, you have to know these following bit patterns and their functionalities.

Bit pattern(operator) and their functionalities.

1100: add next two 4 bits and produce the result after 2 cycles ,stay for 2 cycles

and put 0 for the result(vout).

1101: add next two 5 bits and produce the result after 2 cycles ,stay for 2 cycles

and put 0 for the result(vout).

1111: stop.

You have to implement functionalities for these three bit patterns(1100,1101and 1111).

For other bit patterns, you just ignore and keep examining incoming bit pattern.

|  | Input | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Output | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 0 | 0 | 0 | 0 |  |
|  | Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 |  |

Table 1. Example for the bit pattern  1100.

Operator is 1100, so add incoming next two 4 bits(0001,0001) and produce the output(2).

After that(time 12), keep looking incoming bit patterns starting from the time 13.

Produce output at the time 15 and time 16 and put 0 after that(time 17).

|  | Input | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Output | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 2 | 0 | 0 |  |
|  | Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 |  |

Table 2. Example for the bit pattern  1101.

Operator is 1101, so add incoming next two 5 bits(00001,00001) and produce the output(2).

After that(time 14), keep looking incoming bit patterns starting from the time 15.

Produce output at the time 17 and time 18 and put 0 after that(time 19).

| Color Code |  |  | Operator | Add |
| --- | --- | --- | --- | --- |
|  |  |  | Operand1 | 4bit,5bit |
|  |  |  | Operand2 | 4bit,5bit |
|  |  |  | Time to look the next Operator |  |
|  |  |  | Time for the Output |  |
|  |  |  | Output |  |

// Problem 0:

// Write down your name and your  account  here

// Your Name :  #######

// Your Account: #######

//

// Problem 1:  Convert the following numbers. (15pts)

//    1.1) Decimal     9      to 8-bit Binary: (2pts)

//     Write your answer here;

//   1.2) Decimal    -9      to 8-bit Binary (2pts)

//     Write your answer here:

//    1.3) Decimal    9.375  to Binary (as many bits as needed) (3pts)

//     Write your answer here:

//    1.4) Decimal   -9.375  to IEEE 754 Single Precision (8pts)

//    (Show in hexadecimal):

//   Write your answer here:

//Problem 2:(10pts)

//Look at the incoming bit stream(Input).Then fill out the missing output from time 10 to time 32.

| Input | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Output | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 |

Problem 3(65pts).

//  Complete the module so it can perform the required functionality(65pts).

//  This mini_serial_processing_unit will take bit stream

// and  process the bit stream by looking the operator.

// It  will stop when the operator value is 1111.

//You have to  declare variables and use them.

module mini_serial_processing_unit(vout,v,ck);

input ck;

input v;

//You have to declare variables and their sizes here

//

always @(ck)

begin

///  Write down your code here

//  For better understanding about the output format,

//  Look at the end of this code where the print format is given.

///  There will be no penalty for a little bit longer code

//   as long as your program  produces right answer

endmodule  //End of your solution for the problem 3.

/////     Do not modify this

/////   data memory  for the testbench

/////   you don't have to know these memory and bit_stream_generator module.

/////   these modules  will provide one bit input to the mini_serial_processing_unit module

/////    at every clock.

module memory();

reg [4:0] mem [20:0];

initial

begin

mem[0] =  12;

mem[1] =  7;

mem[2] =  7;

mem[3] =  0;

mem[4] =  0;

mem[5] =  13;

mem[6] =  3;

mem[7] =  12;

mem[8] =  0;

mem[9] =  12;

mem[10] = 1;

mem[11] = 3;

mem[12] = 12;

mem[13] = 0;

mem[14] = 0;

mem[15] = 15;

mem[16] = 0;

mem[17] = 15;

mem[18] = 15;

mem[19] = 0;

end

endmodule

/////   Do not modify this.

/////     bit_stream_generator module

/////    will provide one bit input to the mini_serial_processing_unit module

/////    at every clock.

module bit_stream_generator(v,ck);

input ck;

output v;

reg [5:0] local_v, index;

reg [5:0] cnt;

reg  v;

memory m();

initial index = 0;

initial cnt = 0;

always @(ck)

begin

//#1;

local_v = m.mem[index];

v = local_v[3-cnt];

cnt = cnt+1;

if(cnt == 4) begin

cnt = 0;

index = index +1;

end

if(index ==17) index = 0;

end

endmodule

//

//////////////////////////////////////////////////////////////////////////////////

////////////////////////////Do not modify this test_all module////////////////////////////////////////////////////

module test_all();

integer i,j;

reg ck;

wire  vs;

wire [5:0] outout;

initial  ck = 0;

initial begin

for(i = 1; i<= 40; i = i+ 1)

begin

#10;

ck = ~ck;

end

end

bit_stream_generator b_s_g(vs,ck);

mini_serial_processing_unit m_s_p_u(outout,vs,ck);

endmodule

//###############YOU MAY USE THE FORMAT BELOW FOR YOUR OUTPUT ####

//## Implement this at your mini_serial_processing_unit

//##//###################Print Format##############

//time at  1          input  =      output =

//time at  2          input  =      output =

//   *

//   *

//   *

//

//time at  *   Stop