April 13 ,2005

EE3755 EXAM 2:

Do not turn over the page till I say so.

Some problems are very easy so do not spend

Too much time on them.

If you think a problem is difficult, try to solve

The easy ones First.

Name:

Problem 01(10pts )  03mins  (how much did you spend?      Min)

Problem 02(10pts )  03mins  (how much did you spend?      Min)

Problem 03(10pts )  03mins  (how much did you spend?      Min)

Problem 04(08pts )  04mins  (how much did you spend?      Min)

Problem 05(08pts )  02mins  (how much did you spend?      Min)

Problem 06(06pts )  03mins  (how much did you spend?      Min)

Problem 07(10pts )  05mins  (how much did you spend?      Min)

Problem 08(08pts )  02mins  (how much did you spend?      Min)

Problem 09(15pts )  08mins  (how much did you spend?      Min)

Problem 10(15pts )  15mins  (how much did you spend?      Min)

Bonus(2):   2mins  (Fill out how much did you spend)

Total 102pts; 50 Min.

Problem 1 : Convert the following numbers:

(Estimated Time to Answer(ETA) 3mins: 10pts)

Hint (3/8) = 0.375

a) Decimal  15  to 8-bit Binary: 2pts

00001111

b) Decimal  -15   to 8-bit Binary(2’s complement form):2pts

11110001

c) Decimal 15.375  to Binary (as many bits as needed):2pts

1111.011

d) Decimal    -15.375  to IEEE 754 Single Precision (Show in hexadecimal):4pts

1 10000010 11101100 00000---

1100 0001 0111 0110 00…..

0xc1760000

##For bonus:  How much did you spend?      Mins##

Problem 2:   Fill in the values for x  below.(ETA  3mins: 10 pts) ##############################################

module opp_example();

reg [7:0] x,a;

reg [3:0] b,c,d;

reg [2:0] s;

initial begin

a = 19;

x = a < 10 ? 10 : a < 20 ? 20 : 30;    //x = 20

b= 4'h6;

c= 4'b0001;

x = b & c;                           // x  = 0

x = b && c;                          // x  = 1

b  = 4'b0001;

x = { 3'd2,1'h1,b};                // x = 01010001  =81

x = 8'b00001011;

s = 2;

x = x << s;                          // x = 00101100 =44

b = -3;

c =  1;

x  = b > c ;                    //  x =  1

// explain why?   Negative number = big positive at unsigned 1pts.

b = -1;

c = 3;

x = b <= c;                     // x =           0

b = -1;

c = 15;

x = b === c;                         // x = 1

b = 2;

c = 3;

x = b == c;                           //x = 0

end

endmodule;

##For bonus:  How much did you spend?      Mins##

Problem 3:  Complete the module (ETA 3mins: 10pts):

answer at ##; There are five of them.

module three_bit_Booth(prod,ready,multiplicand,multiplier,start,clk);

input [15:0]  multiplicand, multiplier;

input         start, clk;

output        prod;

output        ready;

reg [32:0]    product;

wire [31:0]   prod = product[31:0];

reg [3:0]     bit;

wire          ready = !bit;

reg           d;

initial bit = 0;

wire [16:0]   multsx = {multiplicand[15],multiplicand};

always @( posedge clk )

if( ready && start ) begin

bit     = 8;

product = { 17'd0, multiplier };

d = 0;

end else if( bit ) begin

case ( {product[1:0],d} )

3'b001: product[32:16] = product[32:16] + multsx;

3'b010: product[32:16] = ##product[32:16] + multsx;

3'b011: product[32:16] = ##product[32:16] + 2 * multiplicand;

3'b100: product[32:16] = ##product[32:16] - 2 * multiplicand;

3'b101: product[32:16] = ##product[32:16] - multsx;

3'b110: product[32:16] = ##product[32:16] - multsx;

endcase

d = product[1];

product = { product[32], product[32], product[32:2] };

bit     = bit - 1;

end

endmodule

##For bonus:  How much did you spend?      Mins##

Problem  4: Complete the module.(ETA  4mins: 8pts)

///Complete carry[0] to carry[4]

// A 5-bit CLA using five cla slice parts.  The generate and propagate

// signals are used to produce the carry  signals.

module cla_5(sum,cout, a,b,cin);

input [4:0] a, b;

input cin;

output [4:0] sum;

output  cout;

wire [4:0]   g, p, carry;

/// Logic for Carry  Signals

###########################

assign       carry[0] =  g[0] | cin & p[0];

assign       carry[1] =  g[1]  |    g[0] & p[1] |

cin & p[0] & p[1];

assign       carry[2] =  g[2]  |    g[1] & p[2] |

g[0] & p[1] & p[2]  |

cin & p[0] & p[1] & p[2];

assign       carry[3] = g[3] |       g[2] &p[3] |

g[1] &p[2] & p[3] |

g[0] & p[1] & p[2] &p[3] |

cin  & p[0] & p[1] & p[2] & p[3];

##############################

assign   carry[4] =     you don’t have 	 to write code  for  this.

assign       sum[0]  =  p[0] ^ cin;

assign      sum[1]  = p[1] ^ carry[0];

assign      sum[2]  = p[2] ^ carry[1];

assign      sum[3]  = p[3] ^ carry[2];

assign      sum[4]  = p[4] ^ carry[3];

assign      cout    = carry[4];

cla_slice_part s0(g[0],p[0],a[0],b[0]);

cla_slice_part s1(g[1],p[1],a[1],b[1]);

cla_slice_part s2(g[2],p[2],a[2],b[2]);

cla_slice_part s3(g[3],p[3],a[3],b[3]);

cla_slice_part s4(g[4],p[4],a[4],b[4]);

endmodule

##For bonus:  How much did you spend?      Mins##

Problem 5:  Name the four basic steps of synthesis(ETA 2mins: 8pts )

(1) Synthesize to RTL (Inference)  (2) Technology Mapping  (3) Optimization

(4) Place and Route

Problem 6: The following module my_loop is not synthesizable and is not working. (ETA 3mins: 6pts)

module my_loop(a,b);

input [15:0] b;

output a;

reg [3:0] a;

reg [3:0] i;

integer s;

initial begin

s = 0;

for(i=0; i<16; i=i+1) s = s + a[i];

a = s;

#5;

end

endmodule

a) why is  the above module not working?

Loop index should be at least 5bits long   reg[4:0] i

b) why is the above module not synthesizable?  Delay initial

##For bonus:  How much did you spend?      Mins##

Problem 7: Draw the figure for the module neq_slice_es and nequal(ETA 5mins 10pts).

module neq_slice_es(eo,a,b,ei);

input a, b, ei;

output eo;

wire   aneb, aeqb;

xor x1(aneb,a,b);

and a1(eo,aneb,ei);

endmodule

module nequal(eq,a,b);

input [1:0] a, b;

output      eq;

wire        e1;

neq_slice_es es1(e1,a[1],b[1],1’b1);

neq_slice_es es0(eq,a[0],b[0],e1);

endmodule

##For bonus:  How much did you spend?      Mins##

Problem 8:     BFA with delays(ETA 2mins: 8 pts).

Suppose "a" changes at t=50 and "b" changes at t=200.

When do sum and cout change?

module bfa_implicit_d(sum,cout,a,b,cin);

input a,b,cin;

output sum,cout;

assign #3 sum =

~a & ~b &  cin |

~a &  b & ~cin |

a & ~b & ~cin |

a &  b &  cin;

assign #2  cout = a & b | b & cin | a & cin;

endmodule

##For bonus:  How much did you spend?      Mins##

sum at 53 ,203

cout at 52,202

Problem 9: Write a verilog program to add these two special format numbers(ETA 8mins:15pts).

There are 2 numbers in 16 bits input.

//X = (A1,A0)

//Y=(B1,B0)

each number is 8 bits long(A1,A0,B1,and B0 are 8bits long)

when we add two numbers  at the same position,

if the result is more than 255 , the result will be set to 255.

This kind of operation is widely used at graphic application.

for example:

input        X   00000001..00100011

input        Y   01110001..11111110

result    X+ Y   01110010..11111111

module brightness(Z,X,Y)

input [15:0] X;

input [15:0] Y;

output [15:0] Z;

//solution comes here//Short code; total length is less or equal to   3 lines

//small deduction for longer code

always @(X or Y) begin

Z[15:8]  = ( X[15:8] + Y [15:8]) >255 ? 255 :  X[15:8] + Y [15:8];

Z[7:0]  =  (X[7:0]  + Y [7:0]  ) >255 ? 255 :  X[7:0] + Y [7:0];

end

endmodule

##For bonus:  How much did you spend?      Mins##

Problem 10 : Complete the module below so that it determines whether its input, a floating point number in IEEE 754 single format, is positive, zero, negative, and whether it is an integer.

Output pos is 1 if the input is positive, neg is 1 if it's negative, etc.

(ETA 15mins: 15pts)

module fp_flags(pos,zero,neg,int,single);

input [31:0] single;

output pos, zero, neg, int;

##For bonus:  How much did you spend?      Mins##

//Solution

reg int;

reg sign;

reg [7:0] exp;

reg [22:0] frac;

reg zero, pos, neg;

reg [5:0] loc;

integer i;

reg found;

always @( single ) begin

sign = single[31];

exp = single[30:23];

frac = single[22:0];

zero = !single[30:0];

pos = !sign && !zero;

neg = sign && !zero;

found = 0;

for(i=0; i<23; i=i+1)if( !found && frac[i] ) begin loc = i; found = 1; end

if( !found ) loc = 23;

int = zero || loc + exp >= 150;

end

endmodule