EE 3755    MIPS 1      Due: TBA

Problem 1: Answer the questions without looking anything.(this is practice, so do not use the simulator program although you may check the correctness of your answer)

LOOP:

0x400000        add $t1, $t2, $t4

addi $t1, $t2,0x20

nop

beq $s0,  $s1, LOOP

(For the problem 1.1)

1.1)Write Machine code for the above 4 instructions.

(Hexadecimal format)

Hint :

opcode for add  is 0 and func field is 0x20.

opcode for addi is 8 and think about rt and rs fields.

opcode for beq  is 4

register number for $t0  is 8

register number for $s0 is 16

What will be the offset value for LOOP  when we are using delayed branch?

What will be the offset value for Loop when we are using non delayed branch?

##############################################

START:

0x400000    J LINE1

Nop

Nop

LINE1       beq $s0,$s1,LINE2

add  $s1,$s2,$s3

LINE2       nop

(For the problem 1.2)

1.2)Write Machine code for the above 5 instructions.

(Hexadecimal format)

Hint :

opcode for j is 2 .

What will be the offset value for LINE1 and LINE2.

1.3)Compute this branch target address:

0x400000    bne r1, r2, 3

1.4)compute this branch target address

0x40000c    bne r1,r2,-3

Problem 2: Briefly explain about three instruction formats for MIPS.

(Less than 20lines.

Your answer should contain at least these following information.

- a)      How big is each field at each format?

- b)      Field position.

Problem 3. write the  register values after executing following instructions.

Loop:

| 0x1000 | Addi r1, r0, 3; | R1= |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Addi r2, r0, 5; | R1= | R2= |  |  |  |
|  | Sub r3, r1, r2; | R1= | R2= | R3= |  |  |
|  | Sub r4, r1, r2; | R1= | R2= | R3= | R4= | R31= |
|  | Beq r3, r0, loop; |  |  |  |  | R31= |
|  | ori r1, r2, 20; | R1= | R2= | R3= |  | R31= |
|  | Addi r4, r0, 0x2000; | R1= | R2= | R3= | R4= | R31= |
|  | jalr r4; | R1= | R2= | R3= | R4= | R31= |

Problem 4: True(right) or False(wrong) instruction .

T      F

4-1)  addi $t0,0x1234,$t0

4-2)  subi $t0,$t1,0x1234

4-3)  jr    $0x123456

4-4)  j     0x123456789

Problem 5:  Briefly explain about the register usage for procedure call.

(Less than 20 lines, please. your answer at least should contain explanation about v0, a0,t0,s0 registers)

Problem 6:

6-1)Write  MIPS programs  for the following c programs ,

..    # REGISTER USAGE ..SUM , $s0, i, $t0,

1)...for(i = 1; i<5 ;++1)

{

sum = sum + i;

}

a) assume Delayed branch without delay slot

b) assume Delayed branch with delay slot

2)...for(i = 1; i<5;i = i+2)

{

sum = sum + i;

sum = sum + i + 1;

}

a) assume Delayed branch without delay slot

b) assume Delayed branch with delay slot

6-2)Count  the total numbers of instructions(running instruction count)  to compute sum for  the 4 programs