EE3755  Homework 4  Due :TBA

Estimated time to finish:

Prob.1:  20Mins.

Prob.2 : 20Mins.

Total :  40Mins.

Use bgx_shell –gui

How to submit: Hard copy during the class.

Problem1. Use the Synthesis program.

1.1) Draw the figure for the module.

1.2) Look at the figure and briefly explain the functionality of the circuit.

module neq_slice_es(eo,a,b,ei);

input a, b, ei;

output eo;

wire   aneb, aeqb;

xor x1(aneb,a,b);

and a1(eo,aneb,ei);

endmodule

module nequal(eq,a,b);

input [3:0] a, b;

output      eq;

wire        e1, e2, e3;

neq_slice_es es3(e3,a[3],b[3],1'b1);

neq_slice_es es2(e2,a[2],b[2],e3);

neq_slice_es es1(e1,a[1],b[1],e2);

neq_slice_es es0(eq,a[0],b[0],e1);

endmodule

Problem2. Use the Synthesis program,

2-1).Draw the figure for the module compare.

( the figure maybe different(from the one at the class)

when you are using bgx_shell -gui?)

2-2). Look at the figure and briefly explain the functionality of the circuit.

module compare(gt, lt, a, b);

input  [2:0] a, b;

output gt, lt;

wire [2:0] a, b;

reg    gt, lt;

integer   i;

always @( a or b ) begin

gt = 0; lt = 0;

for(i=2; i>=0; i=i-1)

if( !gt && !lt ) begin

if( a[i] < b[i] ) lt = 1;

if( a[i] > b[i] ) gt = 1;

end

end

endmodule // compare