EE 3755  Homework 6  Due: TBA(After covering Note 3 and Note4)

MIPS 2:

You should know how to use “xspim”

Use “  main” at the beginning instead of “__start ” and assume no delayed branch when you are using xspim.

Estimated time to finish:

Prob.1 : 5 hours.

Prob.2 :1 hours.

Prob.3: 1 hours.

Total :  7 hours.

When you submit the Hw., Please write down how much time did you spend for the problems.

How to submit:

Hard copy during the class and program on your account and name it( mips1.asm) for problem 1.

Just hard copy for  problem 2 and problem 3.

Problem 1:   (60 pts )  hard copy

and  leave your program on your account and name it mips1.asm .

Write a MIPS program to show word statistics for a paragraph(textdata)

Assume the longest word length is  25

output format:

Occurrences  of  word of  length x:

1 character word  .= 1

2 character word  .= 6.

3 ….

…..

25.character word.  = 0

#############################################################

.data

textdata:  .asciiz  “In a distant galaxy eons before the creation of the mythical planet known as Earth, vast civilizations have evolved, and ruling the galaxy is an interstellar Empire created from the ruins of an Old Republic that held sway for generations\n

Table:    .space  100

#Hint) Words are separated by the space or spaces.

#     ASCII value for the space is 32(decimal) and ASCII value for the null(\n)

#    0(this is for   end of the  sentence).

#       $t0 = location of the string

#       $t1 = hold scanned character

#       $t2 = holds the ascii value for a space(32)

#       $t3 = used for address computation

#       $s0 = holds the word length cnt

#       $s1 = holds the address for TABLE

.data

string:

.asciiz "In a distant galaxy eons before the creation of the

mythical planet known as earth, vast civilizations have eveolved and

ruling the galaxy is an interstellar Empire created from the ruins of an

Old Republic that held sway for generations"

return:

.asciiz "\n"

wordcntmsg:

.asciiz "the word count of the length x \n"

newline:

.asciiz "\n"

wordlength:

.ascii  " character word = "

.align 4

TABLE:  .space 100

.text

.globl main

main:

la $t0, string                  # stores the address for string in $t0

la $s1, TABLE

addi $t2, $0, 32                # stores the ascii value for space in $t2

LOOP:

lbu $t1, 0($t0)                       # load the next charater

beq $t1, $0, DONE               # END of STRING

nop

beq $t1,$t2,RESETWLC            # space

nop

addi $s0,$s0,1                  #inclement word length cnt.

addi $t0,$t0,1                  #point next character.

j LOOP

nop

RESETWLC:

#inclement the word lenght cnt of that word length

# from the memory location.($s1)

# find memory location.

beq $s0,$0,PreviousSF

nop

# address computation

add $t3,$0,$s0  #

addi $t3,$t3,-1

sll $t3,$t3,2   # multiply 4 to get byte address.

add $t3,$t3,$s1 # get momory address.

lw  $t4,0($t3)  # get previous word length cnt from the memory.

addi $t4,$t4,1

sw  $t4,0($t3)  # inclement and store the cnt back.

add  $s0,$0,$0  #reset word length cnt

PreviousSF:

addi $t0,$t0,1  #point next character.

j LOOP

nop

DONE:   # print and Exit..

beq $s0,$0,PRINT

nop

# address computation

add $t3,$0,$s0  #

addi $t3,$t3,-1

sll $t3,$t3,2   # multiply 4 to get byte address.

add $t3,$t3,$s1 # get momory address.

lw  $t4,0($t3)  # get previous word length cnt from the memory.

addi $t4,$t4,1

sw  $t4,0($t3)  # inclement and store the cnt back.

PRINT:

la $a0, wordcntmsg

li $v0, 4

syscall

li $t0,26

li $t1,1

P_LOOP:  move $a0,$t1

li $v0,1

syscall

la $a0,wordlength

li $v0,4

syscall

add $t3,$0,$t1  #

addi $t3,$t3,-1

sll $t3,$t3,2   # multiply 4 to get byte address.

add $t3,$t3,$s1 # get momory address.

lw  $a0,0($t3)  # get previous word length cnt from the memory.

li $v0,1

syscall

la $a0,newline

li $v0,4

syscall

addi $t1,$t1,1

beq $t1,$t0,EXIT

nop

j P_LOOP

nop

EXIT:   addi $v0, $0, 10

syscall                         # Exits program

LB_SUB:                                 #

add $t4, $0, $a0                #

srl $t4, $t4, 2                 #

sll $t4, $t4, 2                 #

sub $t5, $a0, $t4               #

addi $t7, $0, 8                 #

mul $t6, $t5, $t7               #

lw $v0, 0($t4)                  #

addi $t8, $0, 24                #

sub $t8, $t8, $t6               #

sll $v0, $v0, $t8               #

srl $v0, $v0, 24                #

jr $ra                          #

nop

Problem 2: (30 pts)

  - Write  MIPS programs  for the following c programs ,

..    # REGISTER USAGE ..SUM , $s0, i, $t0,

1)...for(i = 1; i<5 ;++1)

{

sum = sum + i;

}

2.1.1.1) assume Delayed branch without delay slot

2.1.1.2) assume Delayed branch with delay slot

2)...for(i = 1; i<5;i = i+2)

{

sum = sum + i;

sum = sum + i + 1;

}

2.1.2.1) assume Delayed branch without delay slot

2.1.2.2) assume Delayed branch with delay slot

2.1.1.2)

li $t0,1

li $t1,5

LOOP: beq $t0,$t1,EXIT_LOOP

nop

add $s0,$s0,$t0

addi $t1,$t1,1

j LOOP

nop

EXIT_LOOP:

# 2.2.1.2)

li $t0,1

li $t1,5

LOOP: beq $t0,$t1,EXIT_LOOP

nop

add $s0,$s0,$t0

addi $t1,$t1,1

add $s0,$s0,$t0

addi $t1,$t1,1

j LOOP

nop

EXIT_LOOP:

2-2)Count  the total numbers of instructions(running instruction count)  to compute sum for  the 4 programs (2.1.1.1, 2.1.1.2,  2.1.2.1 and 2.1.2.2).

Problem 3: Explain following MIPS CODE briefly (10 pts)

What_am_I_doing:

move $t0,$a0

srl  $t0,$t0,2

sll  $t0,$t0,2       #

sub  $t2,$a0,$t0  #

addi $t4, $zero,8

mul  $t3,$t2,$t4   #

lw   $v0,0($t0)

addi $t5,$zero,24

sub  $t5,$t5,$t3

sll $v0,$v0,$t5

srl $v0,$v0,24

jr $ra

nop

3-1)  WHY DOES SOMEONE NEED THIS CODE?(What does  this code do?)

Hint: big endian or little endian.

#  To convert Little Endian format data to Big Endian format data

#  It looks like  lbu $v0,0($a0)

###################### Template for program 1 ######################

#       $t0 = location of the string

#       $t1 = hold scanned character

#       $t2 = holds the ascii value for a space(32)

#       $t3 = used for address computation

#       $s0 = holds the word length cnt

#       $s1 = holds the address for TABLE

.data

string:

.asciiz "In a distant galaxy eons before the creation of the

mythical planet known as earth, vast civilizations have evolved and ruling the galaxy is an interstellar Empire created from the ruins of an Old Republic that held sway for generations"

return:

.asciiz "\n"

wordcntmsg:

.asciiz "the word count of the length x \n"

newline:

.asciiz "\n"

wordlength:

.ascii  " character word = "

.align 4

TABLE:  .space 100

.text

.globl main

main:

## Write your code from here.