Problem 1. Look at the ripple_carry_counter module below

and answer 1.1 , 1.2 and 1.3.

Only part of the testbench is changed.

1.1) When does the simulation end?

a)at 220

b)at 210

c)at 130

d)at 140

e)at 155

f)at 135

g)at 225

h)at 200

1.2)  True or False.

The code at Line 2 in the testbench

is changed from

#15 reset = 1'b0; to

#11 reset = 1'b0;

1.2.1)   The output from L1 to L10(at problem 1.3) will  not be

changed(True, False).  T

1.2.2)   The output at L11(at problem 1.3) will be changed(True, False).

F

1.3) Fill the blank on the output.

0 Output q =0     //L1

(20 )Output q =1       //L2

(30 )Output q =2       //L3

40 Output q =3       //L4

50 Output q =4       //L5

60 Output q =5       //L6

70 Output q =6       //L7

80 Output q =7       //L8

90 Output q =8       //L9

100 Output q =9       //L10

(  110 )Output q =( 10 )    //L11

( 115  )Output q =( 0 )    //L12

(  130 )Output q =( 1 )    //L13

#############################################################

module ripple_carry_counter(q,clk,reset);

output [3:0]  q;

input clk, reset;

T_FF tff0(q[0],clk,reset);

T_FF tff1(q[1],q[0],reset);

T_FF tff2(q[2],q[1],reset);

T_FF tff3(q[3],q[2],reset);

endmodule

module T_FF(q,clk,reset);

output q;

input clk,reset;

wire d;

D_FF dffo(q,d,clk,reset);

not n1(d,q); //not is a Verilog-provided primitive. case sensitive

endmodule

//module D_FF with synchronous reset

module D_FF(q,d,clk,reset);

output q;

input d,clk,reset;

reg q;

always @(posedge reset or negedge clk)

if (reset)

q = 1'b0;

// module D_FF with synchronous reset

else

q = d;

endmodule

// Test bench or Stimulus Block

module stimulus;

reg clk;

reg reset;

wire [3:0] q;

//instantiate the design block

ripple_carry_counter r1(q, clk, reset);

// Control the clk signal that drives the design block.Cycle time =10

initial

clk = 1'b0; //set clk to 0

always

#5 clk = ~clk; // toggle clk every 5 time units

//Control the reset signal that drives the design block

/////////////////This part is different from the notes/////////////////

initial

begin

reset = 1'b1;      //Line 1

#15 reset = 1'b0;  //Line 2

#100 reset = 1'b1; //Line 3

#15 reset = 1'b0;  //Line 4

#10 $finish; //terminate the simulation

end

// Monitor the outputs

initial

$monitor($time, "Output q = %d",q);

endmodule

Problem 2:  Add these two numbers

A = 2.5

B = 1.5.

2.1) Convert A and B to IEEE Double Precision Form

2.2) Add those result from 2.1 in IEEE Double Precision

Show all of your work.(Hexa decimal format)

Solution

2.5 = 0x4020

1.5 = 0x3fc0

4.0 = 0x4080

Problem 3:   Fill in the values for x  below.(ETA  3mins: 10 pts)

module opp_example();

reg [7:0] x,a;

reg [3:0] b,c,d;

reg [2:0] s;

initial begin

a = 19;

x = a < 30 ? 10 : a < 20 ? 20 : 30;    //x = 10

b= 4'h6;

c= 4'b0001;

x = b & c;                           // x  = 0

x = b && c;                          // x  = 1

b  = 4'b0001;

x = { 3'd2,1'h1,b};                // x = 01010001  =81

x = 8'b00001011;

s = 2;

x = x << s;                          // x = 00101100 =44

b = -3;

c =  1;

x  = b > c ;                    //  x =  1

// explain why?   Negative number = big positive at unsigned 1pts.

b = -1;

c = 3;

x = b <= c;                     // x =           0

b = -1;

c = 15;

x = b === c;                         // x = 1

b = 2;

c = 2;

x = b == c;                           //x = 1

end

endmodule;

Problem 4: Precision at IEEE754 FP single.

True or False

4-1)9 decimal digits are enough to represent the fraction field of floating

point.

T    F

4-2)one can represent 0.625 without losing any accuracy.

T    F

4-3)one can represent 0.2 without losing any accuracy.

T    F

4-4)This assignment makes sense:

a=1.23456789;

//  Variable a is represented by using IEEE754 FP single

format.

T    F

4-5)The addition(c = a+b) makes sense:

a = 0.01234;

b = 134567.89;

c = a+b;

// Variable a,b and c are  represented by using IEEE754 FP

single format.

T    F

Solution:

TTFFF

Problem 5: Complete the module below so that it determines whether its

input, a floating point number in IEEE 754 Double format, is positive, zero,

negative, and whether it is an integer.

Output pos is 1 if the input is positive, neg is 1 if it's negative, etc.

(ETA 15mins: 15pts)

module fp_flags(pos,zero,neg,int,double);

input [63:0] double;

output pos, zero, neg, int;

//Solution

reg int;

reg sign;

reg [10:0] exp;

reg [51:0] frac;

reg zero, pos, neg;

reg [7:0] loc;

integer i;

reg found;

always @( doulble ) begin

sign = doulble [31];

exp = doulble [30:23];

frac = doulble [22:0];

zero = ! doulble [30:0];

pos = !sign && !zero;

neg = sign && !zero;

found = 0;

for(i=0; i<52; i=i+1)if( !found && frac[i] ) begin loc = i; found = 1; end

if( !found ) loc = 52;

int = zero || loc + exp >= 1075;

end

endmodule

Problem 6:   Similar example is on the Notes.

6-1)Write down the output from the testbench.

Hint:)  Monitor display the value of variable whenever the value

of the varialbe changes

When does the sum change?

When does the cout change?

module bfa_implicit_d(sum,cout,a,b,cin);

input a,b,cin;

output sum,cout;

assign #3 sum =

~a & ~b &  cin |

~a &  b & ~cin |

a & ~b & ~cin |

a &  b &  cin;

assign #2 cout = a & b | b & cin | a & cin;

endmodule

module testbench();

wire sum,cout;

reg a,b,cin;

bfa_implicit_d  bfa_im(sum,cout,a,b,cin);

initial

begin

a = 0;

cin = 0;

b = 1;

#4;

a = 1;

end

initial

begin

$monitor($time, " Output sum = %d cout =%d",sum,cout);

end

endmodule

########################

solution...

0 Output sum = x cout =x

2 Output sum = x cout =0

3 Output sum = 1 cout =0

6 Output sum = 1 cout =1

7 Output sum = 0 cout =1

#########################

Problem 7:

The output of the module below(this_xor_gate) is not what you are

expecting for an xor gate.

7-1: Write down the testbench output.

7-2: Which line from this_xor_gate should be modified for a correct xor

gate output?

a) L1

b) L2

c) L3

d) L4

e) L5

7-3: What will be the modified code for the solution of 7-2?

7-4: What will be the output of the correctly modified code

(testbench output of a correct xor_gate)?

module this_xor_gate(x,a,b);

input a, b; //L1

output x;   //L2

reg    x;   //L3

initial       //L4

begin

x = a ^ b; //L5

end

endmodule

module testbench();

wire x;

reg a,b;

this_xor_gate tag(x,a,b);

initial

begin

a = 1;

b = 1;

# 10;

a = 1;

b = 0;

end

initial

$monitor($time," Output x = ",x);

endmodule

################

solution

7-1 0 Output x = x

7-2 L4

7-3  always@(a or b)

7-4

0 Output x = 0

10 Output x = 1

###############

Problem 8:   Look at the module below(assignment_practice).

There are some lines which are not following assingment rule.

8-1: Name all of the lines(from L1 to L10) which is not correct.

8-2: For the answer of 8-1,

give the reason why the line is not following

assignment rule.

module assignment_pratice(a,b,c,d,e);

input c,d,e;

output a,b;

reg  f;

reg  e;              // L1

wire h;

assign a  = c & d;

assign f = 1;        // L2

assign h = 1;        // L3

b = d & e;           // L4

endmodule

module testbench();

wire a,b,c,d;

reg e,f,g;

reg h;           //L5

reg x,y,z;

assignment_practice  assign_im(a,h,c,d,e);

assign a = 1;    //L6

initial

begin

d = 0;        //L7

e = 0;        //L8

assign  c = 1; //L9

#10

h = 1;        //L10

end

endmodule

#######################################################

//solution..

//L1 .. e should be wire.

//L2 .. f should be wire or drop assign.

//L4 .. put assign.

//L5 .. h should be net.

//L7 .. can not assign net d in the procedural code.

//L9 .. can not assign net c in the procedural code.

Problem 9:   Drive program for mutiplier

9-1)Write a program(test_memory_s(gl,ck) module) which

will read 1bits from the memory(memory module) at every clock cycle

and test if the number of 1s is greater than the number of  0s

at every  4 cyles.

starting at address 0 and go upto address 15 and go back address 0

again.

(address 0,1...15,0,1,...)

9-2) Write a drive program(testbench) for 9-1.

9-3)Write down first 5 output of testbench2() module.

module memory();

reg [3:0] mem [0:15];

integer    i;

initial

begin

for(i=0;i<16;i = i+1) mem[i] = i;

end

endmodule

module test();

memory m();

initial begin

for(i=0;i<16;i = i+1) $display(" mem[%d] = %b\n",i,m.mem[i]);

end

endmodule

module test_memory_c(same,ck);

input ck;

output same;

reg same;

integer cnt_0,cnt_1,k;

reg [3:0] index,v;

memory m();

initial index = 0;

always @(ck)

begin

cnt_0 = 0;

cnt_1 = 0;

v = m.mem[index];

for( k = 0; k <4;k = k+1)

begin

if(v[k] == 0 ) cnt_0 = cnt_0 + 1;

if(v[k] == 1 ) cnt_1 = cnt_1 + 1;

end

same = 0;

if(cnt_1 == cnt_0) same = 1;

//          if(cnt_1 > cnt_0) gl  = 1;

index = index +1;

end

endmodule

module testbench2();

integer i,j;

reg ck;

wire [3:0] v1;

wire vs;

wire same;

test_memory_c t_c(same,ck);

initial  ck = 0;

initial begin

for(i = 1; i<= 20; i = i+ 1) begin

#1;

ck = ~ck;

end

end

always @(ck)

begin

$display("time = %t , same = %b\n", $time,same);

end

endmodule

////////////////////////////////////

module test_memory_s(gl,ck);

input ck;

output gl;

reg same;

///////////

// your solution comes here

endmodule

module testbench();

integer i,j;

reg ck;

wire [3:0] v1;

wire vs;

wire gl;

initial  ck = 0;

initial begin

for(i = 1; i<= 20; i = i+ 1)

begin

#1;

ck = ~ck;

end

end

////////

// your solution comes here

endmodule

#########################################

solution..

time =                    1 , same = 0

time =                    2 , same = 0

time =                    3 , same = 0

time =                    4 , same = 1

time =                    5 , same = 0

######################################################

Problem 10:  The module below is not synthesizable.

module not_synthesizable_sum(sum,a);

input [7:0] a;

output [10:0] sum;

reg [10:0]    sum;

integer       i;

always @( a ) begin

sum = 0;

for(i=0; i<=a; i=i+1) sum = sum + i;

end

endmodule

10-1) The above module is not synthesizable. Why?(Short answer less than 5

lines)

// loop index is not determined..

10-2) Make the module synthesizable by modifing some code.

//i< = a --->i < 255

10-3) What will be the synthesis output after making it synthesizable?

Draw the output as simple as possible.(I don't want you draw 225 copies

of the unit).

//  255 copy of (sum = sum + i) ..cascaded..