EE 3755    Homework 4      Due: April 30, 2007

Estimated time to solve:

Prob.1       //  pseudo code ;                10   minutes.(10pts)

Prob.2       //  machine code;               10   minutes.(10pts)

Prob.3       //  address computation;        10   minutes.(10pts)

Prob.4       //  solution is given, just run it;  10   minutes.(10pts)

Prob.5       //  write a program               3   hours.  ( 50pts)

Having Script File    5   minutes.(10pts)

When you work at home, installing PCSpim and psfpt    30 minutes.

Uploading to the department computer                   5 minutes.

Total                4 hours and 20 minutes.

How to submit?

// No hard copy.

// Answer to the questions for problem 0,1,2,3  on a file(name it as sol.txt).

// Copy the solution code  for problem 4 to a file(name it as p4.asm),

// Run solution code for problem 4 by  using  xspim  and save the output to p4.txt file

// ( How to save it? Just copy from the output screen)

// Name your program file for  problem 5  as p5.asm,

// and run xspim for  p5.asm  and save the output to p5.txt

//  Name your script file  as  hw4.txt

//  After you have  all  the above files(sol.txt, p4.txt, p5.asm  and p5.txt),

//  run the script  command.

Use “script  hw4.txt “ command // To show your work for  MIPS Homework.

Use “cat  sol.txt” command     // To display  contents of  your solution file.

Use “cat  p4.txt” command      //To display the output of problem4.

Use “cat  p5.asm” command     //To display the contents of your program for the problem 5.

Use “cat  p5.txt” command      //To display the output of problem5.

then stop the script.

Example) SUN>script hw4.txt

SUN>cat sol.txt

SUN>cat p4.txt

SUN>cat p5.asm

SUN>cat p5.txt

SUN>  ( stop script  by pressing CTRL-D)

## Do all your work on sol.txt file for the problem 0 , problem 1, problem 2, and ##problem3.

##  Problem  0 : Write down your name and your  account.

##  Your  Name :  #######

##  Your  Account: #######

## Problem  1(10pts):  Convert  these pseudo instructions into real instructions.

1.1) seq  $s0,$s1,$s2  # seq rdest, rsrc1,rsrc2 : set equal:

1.2) li  $s1,0x1234

## Problem  2(10pts):

LOOP:

0x400000    add $t2, $t2, $t4

addi $t2, $t2,0x20

nop

beq $s1,  $s2, LOOP

(For the problem 1)

Write  Machine code for the above 4 instructions without looking the Book or Notes.

(Hexadecimal format)

Hint :

opcode for add  is 0 and func field is 0x20.

opcode for addi is 8 and think about rt and rs fields.

opcode for beq  is 4

register number for $t0  is 8

register number for $s0 is 16

// Problem  3(10pts):  Assume non-delayed branch.

3.1)Compute this branch target address:

0x400000    bne s0, s1, 2

3.2)compute this branch target address

0x40000c    bne s0,s1,-2

## Do all your work on sol.txt file for the problem 0 , problem 1, problem 2, problem3.

// Problem 4(10pts):

You should know how to use “xspim”

Use “  main” at the beginning instead of “__start ” and assume  non-delayed branch

when you are using xspim or PCSpim.

Write a MIPS program to show word statistics for a paragraph(textdata)

Assume the longest word length is  25:

output format:

Occurrences  of  word  contains :

A  =  6   //{ A, DISTANT, GALAXY, CREATION, MYTHICAL, PLANET} so 6

B  = 1    //{BEFORE} so 1

…..

Z  =

#############################################################

.data

textdata:  .asciiz  “IN A DISTANT GALAXY EONS BEFORE THE CREATION OF THE MYTHICAL PLANET KNOWN \n

Table:    .space  100

#Hint) Words are separated by the space or spaces.

#     ASCII value for the space is 32(decimal) and ASCII value for (\n)  is

#     0  (this is for  the  end of the  sentence).

### solution code for the problem 4 is given below: all you have to do is run

## this code and save the output  to  p4.txt file. By doing this, you will be familiar

##with MIPS simulator(xspim or PCSpim ) and will be ready to solve problem 5.

###############################

# SOLUTION FOR PROBLEM 4.    ##

###############################

#       $t0 = location of the string

#       $t1 = hold scanned character

#       $s2 = holds the ascii value for a space(32)

#       $t3 = used for address computation

#       $s5 = used for starting address of TABLE2 for a word statistics

#       $s4 = used for starting address of TABLE4 for sentence statistics

#       $s0 = holds the word length cnt

#       $s1 = holds the address for TABLE

.data

string:

.asciiz "IN A DISTANT GALAXY EONS BEFORE THE CREATION OF THE MYTHICAL PLANET KNOWN \n"

return:

.asciiz "\n"

newline:

.asciiz "\n"

word_msg:

.asciiz  " number of occurences of words containing \n"

eq_msg:  .ascii  " = "

.align 4

TABLE2: .space 104   # for temporary use

.align 4

TABLE4: .space 104   # for occurences

.text

.globl main

main:

la $t0, string                  # stores the address for string in $t0

la $s5, TABLE2

la $s4, TABLE4

addi $s2, $0, 32                # stores the ascii value for space in $t2

LOOP:

lbu $t1, 0($t0)                       # load the next charater

beq $t1, $0, DONE               # END of STRING

nop

beq $t1,$s2,RESETWLC            # space

nop

addi $s0,$s0,1                  #inclement word length cnt.

########## Goal:  Record the word Statistics  at the TABLE2 #######

### Step1: Capitalize the Character #######

### Step2: Record Statistics  at TABLE2###

####################################

slti  $t2, $t1, 97         # < 'a'   // not a lower case.

bne   $t2, $0, CAPITAL_DONE

slti  $t2, $t1, 123        # 'z' + 1 // 97-122 lower case.

beq   $t2, $0, CAPITAL_DONE # $t2 =1 means , character is lower case.

addi  $t1, $t1, -32

CAPITAL_DONE:

addi  $t1, $t1, -65    # Set $t1 to table index. ( A->0, B->1, etc.)

sltiu $t2, $t1, 26     # If $t1 is >= 26 then it's not a letter.

beq   $t2, $0, NEXT    # Note that comparison above is unsigned.

nop

sll   $t1, $t1, 2      # Scale index.

add   $t5, $s5, $t1    # Add index on to address of first element

add   $t9, $s4, $t1

lw    $t6, 0($t5)

addi  $t6, $t6, 1

sw    $t6, 0($t5)

NEXT:    addi $t0,$t0,1          #point next character.

j LOOP

nop

RESETWLC:

beq $s0,$0,PreviousSF

nop

###  Goal : Update TABLE 4 for occurences of Words Character contain.

###  TABLE2 has Character statistics of a word.

###  Using that statistics, Update TABLE 4.

#### If contents of TABLE2's cell is not zero,

#####Inclement by one the contents of

#### TABLE4's matching cell

# address computation

addi $t6,$0,25   # $t6 = for scanning TABLE2

add  $t5,$0,$0

add  $t9,$0,$0

REPEAT26:

add  $t8,$s5,$t9  # Starting Address of TABLE2

lw   $t7,0($t8)

beq  $t7,$zero,NO_INC

nop

sw   $0,0($t8)

add  $t8,$s4,$t9  # Starting Address of TABLE4

lw   $t7,0($t8)   # inclement and store back to TABLE4

addi $t7,$t7,1    #

sw   $t7,0($t8)   #

NO_INC:  addi  $t5,$t5,1

add   $t9,$0,$t5

sll   $t9,$t9,2

bne   $t6,$t5,REPEAT26

nop

NOCLEAR: nop

PreviousSF:

addi $t0,$t0,1  #point next character.

j LOOP

nop

DONE:   # print and Exit..

beq $s0,$0,PRINT

nop

PRINT:

PW_LOOP_MAIN:

la $a0,word_msg

li $v0,4

syscall

li $t1,0

li $t0,25

PW_LOOP:

move $a0,$t1

addi $a0,$a0,65

li $v0,11

syscall

la $a0,eq_msg

li $v0,4

syscall

add $t3,$0,$t1  #

sll $t3,$t3,2   # multiply 4 to get byte address.

add $t3,$t3,$s4 # get momory address.

lw  $a0,0($t3)  # get Character information.

li $v0,1

syscall

la $a0,newline

li $v0,4

syscall

addi $t1,$t1,1

beq $t1,$t0,EXIT

nop

j PW_LOOP

nop

EXIT:   addi $v0, $0, 10

syscall                         # Exits program

jr $ra                          #

nop

//Problem  5

Use “  main” at the beginning instead of “__start ” and

assume  non-delayed branch  when you are using xspim or pcspim .

Write a MIPS program to show the total number of characters for a paragraph(textdata),

and save it on a file (p5.asm). Run the program and save the output to a file(name it p5.txt).

When textdata is “IN A DISTANT GALAXY EONS BEFORE THE CREATION OF THE MYTHICAL PLANET KNOWN \n” ,

you have to count the number of characters(only alphabetic letters; Do not count  spaces).

(Look at problem4; This one  is a lot easier than problem 4. You can do this by modifying the solution code for the problem 4 ).

The output format is bellow:

The Total Number  of  Character is    :

##########################################################

(For example:

The Total Number  of  Character is  70

#############################################################

.data

textdata:  .asciiz  “IN A DISTANT GALAXY EONS BEFORE THE CREATION OF THE MYTHICAL PLANET KNOWN \n

#Hint)

#     ASCII value for the space is 32(decimal) and ASCII value for (\n) is

#     0 (this is for the end of the sentence).

#    Template for the code will be given below.

###############################

# Template for  PROBLEM 5.    #####

###############################

#       $t0 = location of the string

#       $s2 = holds the ascii value for a space(32)

#       $t3 = used for address computation

.data

string:

.asciiz "IN A DISTANT GALAXY EONS BEFORE THE CREATION OF THE MYTHICAL PLANET KNOWN \n"

return:

.asciiz "\n"

word_msg:

.asciiz  " The Total Number  of  Character is \n"

.align 4

.text

.globl main

main:

##complete the program ####