Louisiana State University

Department of Electrical and Computer Engineering

EE 4780 – Introduction to Computer Vision

Spring 2008

Problem Set 2

Assigned: February 15, 2008

Due: February 25, 2008. Turn in a hardcopy of your solutions to the class. Also email your answer to Problem 1 to the TA, Ming Zhang [mzhang4@lsu.edu].

Collaboration: This problem set can be worked in teams of two. But do not distribute the problems; instead work on both problems together. You still need to turn in individual reports. Include you partners name in the report.

Problem 1: (50 points) Write a Matlab m-file that will do the bilateral filtering. Your m-file should have the following format:

[img_bilateral] = bilateralfilter(img_input, sigma_r, sigma_d, N);

img_input: Input image. (If the input image is a color image, the bilateral filter should be applied to all channels individually.)

sigma_r: Intensity (range) standard deviation parameter.

sigma_d: Spatial (domain) standard deviation parameter.

N: Spatial neighborhood parameter. It controls how many pixels are included in the neighborhood. The size of the neighborhood will be [2N+1 x 2N+1].

img_bilateral: Output image.

Problem 2: (50 points) The histogram of an image is given as below.

0.4        0.6            1

A

A/2

r

p(r)

(a) Draw the histogram equalization function for this image.

(b) What would be new value of a pixel with intensity 0.5 after histogram equalization is applied?

(c) What would be the histogram of the output image when the following transformation is applied to the image?

0.4             0.8      1

1

1/2

r

T(r)

(d) (Bonus 10 points)  If we would like to obtain an image with the following histogram, what should be the mapping applied to the input image.

1

r

T(r)

0.5             1